IFRC WASH TOOL · TROUBLESHOOTING CHART

Troubleshooting chart: consumer water points (taps, kiosks, cattle troughs, meters)

Module 2, Topic 4: Consumer points  ·  Troubleshooting chart  ·  Curriculum pp. 34–35

Potential unexpected problems, their probable causes and possible solutions. Use this chart in training and keep a copy at the site. Seek technical assistance if a problem cannot be solved with these steps.

| Problem | Probable cause | Possible solution |
| --- | --- | --- |
| Leaking service or gate valve | • Worn-out valve | • Replace the valve. Check that the valve is not being used where a tap is more appropriate. |
| Little or no water flowing | • Storage tank empty • Tank outlet pipe blocked • Distribution pipe leaking or broken • Airlock in distribution pipe • Service or gate valve blocked or closed • Meter blocked | • Check the flow in the transmission pipeline. • Check the strainer and outlet valve. • Repair the pipe. • Check/open the nearest air valve. • Remove and clear the valve (replace if necessary). • Remove and clean the sieve/filter in meter. |
| Continuous leak from tap | • Worn-out screw thread in upper tap assembly • Worn-out washer | • Replace the tap. • Replace the washer (see Handout 2). |
| Silty/Muddy water flow | • Silt/mud in tank • Silt/mud in pipes | • De-silt the tank. • Open all washouts. |
| Air coming through meter and tap | • Air in pipe – indicates that tank goes empty | • Check the air valves; check operation of the tank. |
| Hammer noise | • Rapid flow that is turned on/off too rapidly | • Slow opening/closing of the gate valves/taps. |
| Poor water quality | • Contaminated raw water • Poor storage • Leaks in pipelines | • Review the water system from intake to consumer points to check for source of contamination. • Take water-quality sample and analyse to establish the nature of the pollutants and a proper solution. |