IFRC WASH TOOL · TROUBLESHOOTING CHART

Troubleshooting chart: dams and pans

Module 2, Topic 5: Dams and pans  ·  Troubleshooting chart  ·  Curriculum p. 41

Potential unexpected problems, their probable causes and possible solutions. Use this chart in training and keep a copy at the site. Seek technical assistance if a problem cannot be solved with these steps.

| Problem | Probable cause | Possible solution |
| --- | --- | --- |
| Leakage along toe of dam wall | • Poor design and construction | • Monitor for leakages. |
| Water does not last long after end of rainy periods | • Reservoir area has accumulated a significant amount of silt • Erosion of catchment area • Excessive seepage due to pervious soil in reservoir area | • Remove silt from the reservoir area. • Reduce erosion in the catchment area. • Apply and mix in clay, preferably bentonite clay, to impoundment area. |
| No water from outlet | • Outlet pipe blocked | • Clear blockage at the mouth of the draw-off pipe. • Protect the pipe by placing a ballast surround to the mouth of the draw-off pipe. (Note: a blocked pipe through a dam can be very difficult to unblock.) Do NOT remove the pipe. |
| Polluted water | • Livestock in dam or pan • Contamination from catchment area | • Fence the dam or pan. • Control access. • Discourage open defecation in the catchment area. |
| Excessive weed growth | • High nutrient concentration in water | • Address source of nutrients, possibly by controlling access to the dam or pan or catchment area by livestock. |