IFRC WASH TOOL · TROUBLESHOOTING CHART

Troubleshooting chart: pipelines

Module 2, Topic 2: Pipelines  ·  Troubleshooting chart  ·  Curriculum p. 25

Potential unexpected problems, their probable causes and possible solutions. Use this chart in training and keep a copy at the site. Seek technical assistance if a problem cannot be solved with these steps.

| Problem | Probable cause | Possible solution |
| --- | --- | --- |
| Numerous repeated breakages in pipeline | • Pipeline laid too shallow and pipe exposed to traffic loading • Wrong type of pipe | • Consider relocating or replacing the pipeline. (Consult with a water engineer, qualified water technician or plumber.) |
| No water in pipe | • Breakage, burst • Blockage – air lock • Closed valve • Source is dry • Design problem | • Check pipeline carefully, starting at the intake level and check at various points whether water is reaching each point. (Consult with a water engineer or qualified water technician.) |
| Poor water quality | • Polluted raw water • Burst that has introduced pollutants or sediments • Faulty treatment | • Investigate the problem and identify a suitable solution. Carry out a water quality test if appropriate. |
| Low pressure | • High friction losses from movement of the pipework, restriction in pipes, bursts, low abstraction, excessive consumption | • Identify whether a maintenance solution will resolve low pressure; this may include water rationing, or the problem is related to a design issue. (Consult with a water engineer or qualified water technician.) |