IFRC WASH TOOL · TROUBLESHOOTING CHART

Troubleshooting chart: roof catchment systems for rainwater harvesting

Module 2, Topic 10: Roof catchment systems for rainwater harvesting (RWH)  ·  Troubleshooting chart  ·  Curriculum p. 65

Potential unexpected problems, their probable causes and possible solutions. Use this chart in training and keep a copy at the site. Seek technical assistance if a problem cannot be solved with these steps.

| Problem | Probable cause | Possible solution |
| --- | --- | --- |
| Water does not enter tank | • Blockage in downpipe • Gutter is not installed at a gradient to allow water to flow to the tank; gutter sag | • Remove the blockage. • Clean or replace the screens. • Check the levels on the gutter and reset levels if required; put more brackets on the gutter. |
| No water in tank | • Leaky tap • Leaky tank • Overuse | • Check for and repair any leaks. • Monitor usage. |
| Water does not last long after periods of rain | • Storage volume is low compared to consumption | • Regulate consumption. • Provide additional storage if more water can be harvested. However, check whether the tank usually overflows. If not, then additional roof area and storage are required. |
| Smelly water | • Organic matter in tank decomposing | • Drain and clean the tank |