IFRC WASH TOOL · TROUBLESHOOTING CHART

Troubleshooting chart: sand dams and subsurface dams

Module 2, Topic 6: Sand dams and subsurface dams  ·  Troubleshooting chart  ·  Curriculum p. 45

Potential unexpected problems, their probable causes and possible solutions. Use this chart in training and keep a copy at the site. Seek technical assistance if a problem cannot be solved with these steps.

| Problem | Probable cause | Possible solution |
| --- | --- | --- |
| Water in sand dam but nothing in the well | • No connection between sand dam and shallow well • Blockage of filter drain to shallow well | • Excavation and repair of filter drain. |
| No water in the sand dam | • Lack of inflow • Seepage under or around the wall of the sand dam • Faulty or leaking draw-off pipe | • Check for seepage path and take measures to prevent seepage by sealing the flow path with clay or waterproof plaster on upstream face of wall. • Fix or replace the faulty or leaking draw-off pipe |
| No water in the sand dam | • Dam full of silt not sand • Dam may be incorrectly sited in a watercourse with insufficient sandy material or the wall was raised too quickly during construction that can result in a top layer of silt which prevents water seeping into the lower sandy material | • Remove silt if the watercourse contains sufficient sandy material. |
| Wall at risk from erosion weakening the wall | • Turbulent and erosive water spilling over the wall • Lack of sufficient protection for toe of wall | • Place grouted riprap (loose stone to form a structure that breaks up water flow) to create a protective apron on downstream toe of wall. |
| Polluted water | • Livestock or human activities polluting surface area of dam, resulting in contamination of the water | • Control access by establishing clear rules of access and behaviour and monitoring compliance to the rules. |
| No sand | • Sand harvested for construction purposes | • Control access through establishing clear rules of access and regulate quantity of sand harvested from the sand dam. |