IFRC WASH TOOL · TROUBLESHOOTING CHART

Troubleshooting chart: shallow wells

Module 2, Topic 8: Shallow wells  ·  Troubleshooting chart  ·  Curriculum p. 56

Potential unexpected problems, their probable causes and possible solutions. Use this chart in training and keep a copy at the site. Seek technical assistance if a problem cannot be solved with these steps.

| Problem | Probable cause | Possible solution |
| --- | --- | --- |
| Collapsing well | • Poor construction – usually insufficient well lining and well head | • Repair or improve the well lining, head wall and the apron. |
| Well goes dry | • Water level falls • Water is extracted faster than the recharge rate | • Deepen the well – there is a limit as to how deep a well can be safely excavated. This depends on the surrounding material. |
| Well washed out by floods (in cases where well is placed in riverbed) | • Placing a well in a watercourse is risky and prone to damage due to the turbulence and force of the flood waters, and objects (e.g., logs, rocks) carried by the flood waters | • Seek a safer place in which to construct the well – usually at the edge of the watercourse. • Alternatively, protect The well from flood water by building a hydrodynamic well head. |
| Well silted due to flooding | • Wrong placement of well in flood-prone area or insufficient height on the well head to prevent runoff inflow | • Raise the well head above flood level |