IFRC WASH TOOL · TROUBLESHOOTING CHART

Troubleshooting chart: solar-powered pump systems

Module 2, Topic 13: Solar-powered pump systems  ·  Troubleshooting chart  ·  Curriculum p. 92

Potential unexpected problems, their probable causes and possible solutions. Use this chart in training and keep a copy at the site. Seek technical assistance if a problem cannot be solved with these steps.

| Problem | Probable cause | Possible solution |
| --- | --- | --- |
| Low or no power, or slow motor (less water is being pumped) | • Poor electrical connection due to dirt • Wet or corroded terminals • Insufficient sunlight • Dusty solar panels | • Clean, dry or replace the terminals. • Remove any dust. |
| System switching on and off | • Inverter or regulator poorly installed | • Install away from sunlight and water. |
| Motor stops | • Worn-out brushes | • Replace the brushes. |
| Water leaking from discharge head (mono pump) | • Leaking pump gland Seal | • Tighten the pump gland nuts slightly (do not over-tighten). |