IFRC WASH TOOL · TROUBLESHOOTING CHART

Troubleshooting chart: storage tanks

Module 2, Topic 3: Storage tanks  ·  Troubleshooting chart  ·  Curriculum p. 30

Potential unexpected problems, their probable causes and possible solutions. Use this chart in training and keep a copy at the site. Seek technical assistance if a problem cannot be solved with these steps.

| Problem | Probable cause | Possible solution |
| --- | --- | --- |
| Overflow from tank | • Failure of float valve • Gate valve blocked (outlet side) • Airlock in pipeline (outlet side) | • Replace the rubber washer (see Handout 2). • Replace the pin. • Remove and clear the valve (replace if necessary). Check/ open the nearest air valve. |
| Leaks from tank wall | • Crack in wall | • Repair on the inside – this will depend on the type of tank. |
| Cracks in roof | • Settlement of roof • Too much loading on roof | • Seal all cracks; add additional support if necessary. |