IFRC WASH TOOL · TROUBLESHOOTING CHART

Troubleshooting chart: wind pumps

Module 2, Topic 12: Wind pumps  ·  Troubleshooting chart  ·  Curriculum p. 83

Potential unexpected problems, their probable causes and possible solutions. Use this chart in training and keep a copy at the site. Seek technical assistance if a problem cannot be solved with these steps.

| Problem | Probable cause | Possible solution |
| --- | --- | --- |
| Wind pump does not self-furl | • Furling mechanism not working | • Repair the furling system. |
| Windmill working but no water being raised | • Borehole has run dry • Pump washers leaking • Leaky riser | • Remove the pump and inspect or repair. Use the dipper to check the water level in the borehole. |